TEAM ROCKET

LAYER 1 — AGRONOMIC ENGINE

Final Implementation Specification

FAO-56-compatible reference ET + crop coefficient + root-zone water balance + irrigation trigger and sizing

| Purpose | Implementation-ready specification for the deterministic agronomic layer of AquaSense AI. |
| --- | --- |
| Status | LOCKED BASELINE — use this as the reference design for prototype implementation. |
| Scope | Layer 1 only. ML prediction / Layer 2 and fusion are intentionally outside this document. |
| Primary principle | Use agricultural science to compute a transparent baseline; use real-time soil sensing to update and verify root-zone state. |

Important implementation rule Layer 1 is deterministic. It is not trained like an ML model. Crop coefficients, soil hydraulic parameters, root-zone depth and management parameters are configured/calibrated from agronomic sources and field measurements. The real-time sensor measurement is used to update the root-zone state.

# 1. Locked Layer-1 architecture

Figure 1. Locked Layer-1 architecture (deterministic agronomic + root-zone water-balance engine).

ET0 is computed from climate/weather data using the FAO Penman-Monteith reference method.

ETc is computed from ET0 and stage-specific crop coefficient Kc.

The root-zone state is represented using field capacity, wilting point, rooting depth, total available water (TAW), readily available water (RAW), and live volumetric soil-water content.

Irrigation is triggered when current root-zone depletion reaches the RAW threshold.

The irrigation amount is calculated from the current root-zone deficit and converted to gross applied depth, volume and valve runtime.

Flow/pressure and post-irrigation soil response are used to verify actual delivery and update the root-zone state.

# 2. Inputs and configuration

| Category | Variable | Symbol | Unit | Source / role |
| --- | --- | --- | --- | --- |
| Weather | Air temperature | T | °C | Weather station / API |
| Weather | Relative humidity | RH | % | Weather station / API |
| Weather | Wind speed at 2 m | u2 | m/s | Anemometer / weather API |
| Weather | Solar / net radiation | Rs / Rn | MJ/m²/day | Radiometer or derived from weather data |
| Weather | Rainfall | P | mm | Rain gauge / weather API |
| Location | Elevation | z | m | Static site metadata; used for pressure if not measured |
| Soil | Current root-zone volumetric water content | θcurrent | m³/m³ | Live soil-moisture sensor |
| Soil | Field capacity | θFC | m³/m³ | Soil characterization / lookup + calibration |
| Soil | Wilting point | θWP | m³/m³ | Soil characterization / lookup + calibration |
| Soil | Effective root-zone depth | Zr | m | Crop + local root profile |
| Crop | Crop type | crop | — | Configuration |
| Crop | Growth stage | stage | — | Configuration / phenology tracking |
| Crop | Crop coefficient | Kc | dimensionless | FAO stage table + interpolation |
| Crop | Baseline depletion fraction | ptable | 0–1 | FAO crop/depletion table |
| Management | Irrigation efficiency | Ea | 0–1 | System calibration |
| Hydraulics | Zone area | A | m² | Field/network configuration |
| Hydraulics | Measured flow rate | Q | L/min | Flow sensor; used for runtime |

Prototype sensor minimum For a complete ET0 calculation, the weather input set must provide air temperature, humidity, wind speed and radiation (or enough information to derive radiation), with site elevation/location as configuration. Soil moisture is required for real-time root-zone state. Rainfall, flow and pressure are supporting inputs for the closed-loop irrigation system.

# 3. Equation set — final baseline

All equations below are written so the implementation can use daily time steps as the primary decision interval. Sub-daily calculations can be added later without changing the architecture.

## 3.1 Atmospheric variables

es = 0.6108 × exp(17.27T / (T + 237.3)) Saturation vapour pressure, es in kPa; T in °C.

ea = es × RH / 100 Actual vapour pressure, ea in kPa.

VPD = es − ea Vapour-pressure deficit, kPa.

Δ = 4098 × es / (T + 237.3)² Slope of saturation vapour-pressure curve, kPa/°C.

P = 101.3 × ((293 − 0.0065z) / 293)^5.26 Atmospheric pressure estimate when pressure is not measured; P in kPa, z in m.

γ = 0.000665 × P Psychrometric constant, kPa/°C.

## 3.2 Reference evapotranspiration — ET0

ET0 = [0.408Δ(Rn − G) + γ × 900/(T+273) × u2 × (es−ea)] / [Δ + γ(1 + 0.34u2)] FAO Penman-Monteith daily reference ET. ET0 in mm/day; Rn and G in MJ/m²/day; u2 in m/s.

Prototype convention: for a daily timestep, G is initially approximated as 0 unless the implementation explicitly models daily soil heat flux.

## 3.3 Crop evapotranspiration — ETc

ETc = Kc × ET0 Single-Kc approach is the locked Layer-1 baseline.

Kc is selected by crop and growth stage. During transition stages, Kc is linearly interpolated between stage values.

Kc(t) = Kc,start + ((t − t0)/duration) × (Kc,end − Kc,start) Use only for stage transitions where a linear change is a reasonable approximation.

## 3.4 Root-zone capacity

TAW = 1000 × (θFC − θWP) × Zr TAW in mm; θFC and θWP are volumetric fractions; Zr in m.

p = clip[ptable + 0.04 × (5 − ETc), 0.1, 0.8] ETc in mm/day; p is the allowable depletion fraction.

RAW = p × TAW RAW in mm.

## 3.5 Real-time soil state

S = 1000 × (θcurrent − θWP) × Zr Current root-zone plant-available water storage, S in mm, clipped to [0, TAW].

Dr = TAW − S Current root-zone depletion, Dr in mm.

Dr = clip[1000 × (θFC − θcurrent) × Zr, 0, TAW] Equivalent direct formula for current depletion.

θcrit = θFC − p × (θFC − θWP) Critical volumetric moisture corresponding to RAW threshold.

## 3.6 Trigger logic

If Dr ≥ RAW → irrigation trigger; otherwise → continue monitoring The trigger decision is distinct from the irrigation amount calculation.

## 3.7 Root-zone water balance / state update

Dr,t = Dr,t−1 + ETc,t − Pe,t − Inet,t + DPt + ROt Depletion-form water balance. Dr in mm; Pe = effective precipitation; Inet = net irrigation entering root zone; DP = deep percolation; RO = runoff.

For the prototype, live soil-moisture measurements are the authoritative state correction. The water-balance equation is used to propagate the state between sensor observations and to reason about rainfall/irrigation contributions. Do not double-count rainfall or ETc when a measured post-event soil state is available.

## 3.8 Irrigation amount

Inet = Dtarget Use Dtarget as the planned reduction in root-zone depletion. Prototype baseline: refill toward field capacity, so Dtarget = Dr when a trigger occurs.

Igross = Inet / Ea Gross applied irrigation depth, mm. Ea is system/application efficiency 0–1.

V(L) = Igross(mm) × A(m²) Water volume. 1 mm over 1 m² equals 1 litre.

tvalve(min) = V(L) / Q(L/min) Valve runtime using measured/estimated zone flow rate.

For variable flow, replace the simple runtime formula with volume integration: V = ∫Q(t)dt, and stop when cumulative delivered volume reaches the target.

# 4. Decision logic and state machine

Figure 2. Layer-1 irrigation trigger state machine.

# 5. Implementation sequence

1. Ingest the daily weather inputs and live root-zone soil moisture.

2. Validate units, timestamps, sensor ranges and missing values.

3. Calculate es, ea, VPD, Δ and γ.

4. Calculate ET0 using FAO Penman-Monteith.

5. Retrieve/interpolate Kc from the crop × growth-stage configuration.

6. Calculate ETc = Kc × ET0.

7. Calculate TAW from θFC, θWP and Zr.

8. Retrieve ptable and apply the ETc adjustment to obtain p; calculate RAW.

9. Convert the live soil-moisture reading to root-zone storage S and depletion Dr.

10. Update the root-zone state using the water-balance logic; use the live sensor to correct drift in the state estimate.

11. If Dr < RAW, keep the zone in monitoring state.

12. If Dr ≥ RAW, calculate the planned net irrigation depth, then gross depth, volume and valve runtime.

13. Issue the valve command subject to safety constraints (maximum runtime, tank status, pressure limits, connectivity state).

14. Track actual flow/pressure during the irrigation event and stop when the target cumulative volume is reached or a safety fault occurs.

15. Log predicted quantities, actual delivery and post-irrigation soil state for later analysis and for Layer 2 data generation.

# 6. Diagnostic graphs / expected behavior

The following figures are illustrative diagnostics for implementation and testing. They are not field measurements and must not be presented as measured prototype performance.

Figure 3. Illustrative soil-moisture trajectory showing FC, RAW trigger threshold and WP.

Figure 4. Illustrative daily ETc, rainfall and irrigation accounting.

# 7. Configuration tables required by the implementation

## 7.1 Crop configuration

| Field | Example / description |
| --- | --- |
| crop_id | Unique crop identifier |
| crop_name | Human-readable crop name |
| stage_sequence | Initial / development / mid / late |
| Kc_initial | FAO/reference value |
| Kc_mid | FAO/reference value |
| Kc_end | FAO/reference value |
| stage_durations | Days or phenology-derived stage lengths |
| p_table | Baseline allowable depletion fraction |
| root_depth | Effective rooting depth by stage or crop profile |

## 7.2 Soil / zone configuration

| Field | Example / description |
| --- | --- |
| zone_id | Unique irrigation zone |
| soil_texture | Configured soil class / lab characterization |
| theta_FC | Field-capacity volumetric water content |
| theta_WP | Wilting-point volumetric water content |
| Zr | Effective root-zone depth |
| zone_area_m2 | Irrigated zone area |
| Ea | Application/system efficiency |
| Q_expected | Nominal flow rate; overwritten by measured flow when available |
| pressure_limits | Safe operating pressure limits |

# 8. Required database/logging schema

Every irrigation event should create a complete, auditable record. The same log will later form the local field dataset for Layer 2.

| Timestamp / ID | Field / variable | Purpose |
| --- | --- | --- |
| timestamp | Event / sensor time | Sequence ordering |
| zone_id | Irrigation zone | Spatial identity |
| crop_id / stage | Crop context | Kc/p selection |
| T / RH / u2 / Rn | Weather inputs | ET0 calculation |
| rainfall | Rain event | Water-balance accounting |
| theta_current | Live soil moisture | Root-zone state |
| theta_FC / theta_WP / Zr | Soil parameters | TAW / depletion |
| Kc / ET0 / ETc | Calculated agronomic variables | Model traceability |
| TAW / p / RAW | Calculated thresholds | Trigger traceability |
| Dr_before | Pre-event depletion | Trigger state |
| Inet / Igross / V_target | Irrigation target | Decision output |
| Q(t) / pressure(t) | Actual hydraulic response | Delivery verification |
| V_delivered | Integrated actual volume | Closed-loop verification |
| theta_after | Post-event soil state | State correction / calibration |
| fault_code / override | Exceptions / human intervention | Safety + auditability |

# 9. Validation and acceptance criteria

Unit validation: every equation must be unit-consistent and produce physically plausible ranges.

Sensor validation: θ must remain within plausible [0, 1] volumetric bounds; outliers must be flagged before use.

ET0 validation: compare calculated ET0 against an independent reference/source for selected periods.

Kc validation: verify crop/stage tables and interpolation against the chosen agronomic reference.

Water-balance conservation check: for controlled experiments, cumulative inputs − outputs must approximately match measured soil-storage changes within measurement uncertainty.

Trigger validation: verify that irrigation is not triggered while depletion remains below RAW and is triggered when depletion reaches RAW.

Delivery validation: commanded volume must be compared with measured cumulative flow; define a maximum allowed delivery error during prototype testing.

Post-event validation: after irrigation, the measured root-zone state should move toward the target state without exceeding safety limits.

Failure-mode validation: test missing weather data, bad soil sensor, GSM outage, low pressure, blocked flow and manual override.

# 10. Locked design decisions for prototype V1

| Decision | Locked choice | Reason |
| --- | --- | --- |
| ET method | FAO Penman-Monteith reference ET0 | Transparent, standard agronomic baseline |
| Crop ET | Single-Kc ETc = Kc × ET0 | Simpler to validate first; stage-specific Kc retained |
| Root-zone state | Live θ + FC/WP/Zr | Direct field observability |
| Depletion threshold | RAW = p × TAW | Directly connected to crop stress threshold |
| p adjustment | FAO-style ETc adjustment with bounds | Captures demand-dependent allowable depletion |
| Trigger | Dr ≥ RAW | Clear, auditable scheduling rule |
| Irrigation sizing | Refill toward field capacity for prototype baseline | Simple deterministic target for initial hardware validation |
| Water volume | Depth × zone area | Simple physical conversion |
| Valve runtime | Target volume ÷ measured flow; integrate Q(t) when variable | Connects agronomic requirement to hardware |
| State feedback | Live soil moisture is authoritative correction after events | Prevents model drift / double-counting |
| Layer 1 learning | None | All parameters are configured/calibrated, not ML-trained |

What Layer 1 is not Layer 1 does not choose between LSTM, XGBoost or another ML model. Layer 2 will do that. Layer 1 provides a transparent, scientifically grounded baseline and a real-time root-zone state against which Layer 2 can later be evaluated.

# 11. Known refinements reserved for later

Dual crop coefficient (Kcb + Ke) for separating transpiration and soil evaporation.

Explicit soil hydraulic functions / moisture-release curves instead of lookup-based θFC and θWP.

Detailed infiltration/runoff/deep-percolation models for rainfall events.

Dynamic water-stress coefficient Ks where the crop is already stressed.

Remote-sensing assimilation / spatialized ET or soil moisture.

Sub-daily ET0 and high-frequency control if the final field hardware supports it.

Irrigation strategy optimization beyond full-replenishment-to-FC, once prototype evidence and water-use objectives are established.

# 12. Reference basis

[1] FAO — Chapter 1: Introduction to evapotranspiration. — Reference ET0 is a climatic parameter; FAO Penman-Monteith is the standard method; crop coefficients relate ETc to ET0.
https://www.fao.org/4/X0490E/x0490e04.htm

[2] FAO — FAO Irrigation and Drainage Paper 56: Crop evapotranspiration. — Core equations for ET0, Kc, TAW and RAW; stage-dependent crop coefficients and soil-water stress framework.
https://www.fao.org/4/X0490E/x0490e00.htm

[3] FAO — Chapter 8: ETc under soil water stress conditions. — TAW, RAW, p and crop-water stress logic.
https://www.fao.org/4/X0490E/x0490e0e.htm

[4] FAO AQUASTAT — Irrigation water requirement. — Relationship among ETc, effective precipitation and soil-water storage.
https://www.fao.org/aquastat/en/data-analysis/irrig-water-use/irrig-water-requirement/index.html

[5] FAO — Chapter 4: Irrigation water needs. — Crop water need, effective rainfall and irrigation water need.
https://www.fao.org/4/s2022e/s2022e04.htm

[6] FAO — 2025 Revised Edition: Crop evapotranspiration — Guidelines for computing crop water requirements. — Major update to FAO Irrigation and Drainage Paper 56; foundational concepts retained with enhanced methods.
https://agris.fao.org/search/es/records/6a32b9d6b9a0a56f302a5e20

# 13. Implementation handoff checklist

Create crop table with Kc by stage and p_table values from the selected agronomic reference.

Create soil/zone table with θFC, θWP, Zr, zone area and application efficiency.

Implement unit-tested functions for each equation before integrating sensors.

Implement a root-zone state object containing S, Dr, TAW, RAW and timestamp.

Implement sensor-state correction so measured θ can reset/correct propagated water-balance state.

Implement trigger and irrigation-volume calculation as deterministic, auditable code.

Implement flow-based closed-loop valve stopping logic.

Log every intermediate calculation, not just final valve commands.

Run a controlled single-zone bench experiment before adding Layer 2.

After Layer 1 passes validation, begin Layer 2 model selection and training.

END OF LAYER 1 LOCK — TEAM ROCKET