# Team Rocket — Layer 2
XGBoost Residual Forecasting Engine

Implementation Specification • SIH 2026 • Prototype V1

Locked architecture: Layer 2 is a data-driven correction layer on top of the deterministic Layer 1 agronomic/root-zone water-balance model. The primary model is XGBoost regression. It predicts the 24-hour-ahead residual between observed root-zone moisture and the Layer 1 physics-forward forecast. Layer 2 does not directly command the valve.

# 1. What Layer 2 Does

At decision time t, Layer 1 runs forward for 24 hours using the current measured root-zone state and forecast weather. That produces a physics-only prediction of root-zone volumetric water content at t+24 h. When the actual t+24 h sensor value becomes available, the residual is computed. XGBoost learns the systematic, feature-dependent residual. The corrected forecast is then the physics forecast plus the learned residual.

Physics forecast:   theta_phys(t+24) = Layer1(current state, forecast weather)
Observed:           theta_obs(t+24)
Residual target:    r(t+24) = theta_obs(t+24) - theta_phys(t+24)
XGBoost:             r_hat(t+24) = XGB(features available at t)
Final forecast:      theta_hat(t+24) = theta_phys(t+24) + r_hat(t+24)

# 2. Decisions to Lock Before Coding

| Decision | Locked choice | Reason / boundary |
| --- | --- | --- |
| Model | XGBoost Regressor | Primary Layer-2 model; simple enough for prototype, strong for nonlinear tabular data. |
| Target | 24 h root-zone moisture residual | Measurable target; keeps Layer 1 as agronomic authority. |
| Forecast horizon | 24 h | Operationally useful and achievable for V1; later horizons are optional. |
| Sampling | 1-hour canonical data record | Enough temporal resolution without creating unnecessary data volume. |
| Weather forecast | Required | The model must see future weather information; otherwise it is only trend extrapolation. |
| Physics baseline | Layer 1 forward simulation with forecast weather | Required benchmark and source of residual target. |
| ML role | Predict correction only | No direct GPIO/valve command from XGBoost. |
| Synthetic Stage 1 | Structured feature-correlated bias, not pure random noise | Ensures a learnable residual exists; validates methodology only. |
| Real Stage 2 | Real sensor/irrigation observations | Required before claiming field performance. |
| Decision authority | Layer 1 + deterministic control layer | ML informs future state; safety remains deterministic. |

# 3. End-to-End Architecture

CURRENT FIELD STATE
                               |
                               v
                 +---------------------------+
                 |       LAYER 1             |
                 | FAO-56-compatible engine  |
                 +-------------+-------------+
                               |
                               | current state + forecast weather
                               v
                 +---------------------------+
                 | 24-h PHYSICS FORWARD RUN  |
                 | predicted theta_phys(t+24)|
                 +-------------+-------------+
                               |
                     +---------+---------+
                     |                   |
                     |                   |
             actual theta(t+24)    forecast features at t
                     |                   |
                     +---------+---------+
                               |
                               v
                 residual = actual - physics
                               |
                               v
                 +---------------------------+
                 |        XGBoost             |
                 | residual correction model  |
                 +-------------+-------------+
                               |
                      residual prediction
                               |
                               v
             theta_hat = theta_phys + residual_hat
                               |
                               v
                 +---------------------------+
                 | Layer 1 decision logic    |
                 | irrigation trigger/volume |
                 +-------------+-------------+
                               |
                               v
                            VALVE
                               |
                               v
                            FIELD
                               |
                               +------> DATABASE ------> future training/evaluation

# 4. Feature Contract

Every feature must satisfy one rule: at prediction time t, the value must be genuinely available, measured, configured, derived from past observations, or obtained from a forecast that was available at t.

| Group | Features | Timing |
| --- | --- | --- |
| Current field state | Root-zone moisture; depth-wise moisture; soil temperature if available | Observed at t |
| Historical soil | SM lags 1h, 3h, 6h, 12h, 24h; rolling mean/std; 6h/24h trend | <= t |
| Observed weather | Temperature, RH, wind, radiation, recent rainfall | <= t |
| Historical weather | Rainfall sums 6h/24h/72h; recent temperature/humidity/wind/radiation aggregates | <= t |
| Agronomic | ET0, ETc, Kc, crop, growth stage, root-zone depth, soil parameters | Known/derived at t |
| Irrigation history | Irrigation volume 6h/24h/72h; time since irrigation; flow-confirmed volume; pressure statistics | <= t |
| Forecast weather | Rain 0–6h/6–12h/12–24h; temperature stats; humidity; wind; radiation; forecast ET0 if available | Forecast available at t |
| Calendar | Hour of day, day of year/growth stage | Known at t |

No leakage: Never use future observed rainfall, future observed soil moisture, future actual irrigation, or future sensor measurements as features. Forecast values are allowed only when they are genuinely available from the forecasting service at the prediction timestamp.

# 5. Weather Forecast Dependency

The forecast feed is a hard dependency of the 24-hour prediction design. The implementation should use a provider abstraction so the ML pipeline is not hard-coded to one service. For the prototype, an API such as Open-Meteo can be used to obtain hourly forecast weather; the provider must supply the variables needed by Layer 1 and the XGBoost forecast features.

| Forecast feature | Preferred representation |
| --- | --- |
| Rainfall | 0–6h, 6–12h, 12–24h totals; 24h total |
| Temperature | Mean/min/max for 0–24h and, where useful, 6–12h blocks |
| Relative humidity | Mean/min/max or block means |
| Wind speed | Block means and/or maximum |
| Radiation | Block totals/means consistent with Layer 1 units |
| Forecast ET0 | 24h or block totals when the provider can supply a suitable consistent estimate |

forecast_timestamp = t
forecast_horizon = [t, t+24h]

# Example validation rule
if forecast_data.timestamp < t:
    mark_forecast_stale()

if required_forecast_fields_missing:
    use_fallback_mode()
    flag_prediction_as_degraded()

Fallback: A missing/stale forecast must never silently be treated as valid. For V1, the system should flag degraded prediction quality and rely on the deterministic Layer 1/current-state safety path rather than allow a stale forecast to drive an automatic decision.

# 6. Dataset Design

Canonical frequency: one row per zone per hour. The target for timestamp t is computed from the observation at t+24h. The dataset should retain both raw measurements and derived features so feature generation can be audited.

| Column | Type / Unit | Description |
| --- | --- | --- |
| timestamp | datetime, timezone-aware | Prediction timestamp t |
| zone_id | categorical | Irrigation/field zone |
| crop_id | categorical | Crop identity |
| growth_stage | categorical | Current growth stage |
| theta_rz | m3/m3 | Current root-zone volumetric water content |
| theta_depth_i | m3/m3 | Depth-specific sensor reading, if used |
| temp_c | deg C | Current air temperature |
| rh_pct | % | Current relative humidity |
| wind_ms | m/s | Current wind speed |
| rad_mj_m2_d | MJ/m2/day or canonical hourly equivalent | Radiation input |
| rain_mm | mm | Current/recent rainfall |
| et0_mm | mm | Layer 1 ET0 |
| etc_mm | mm | Layer 1 ETc |
| kc | dimensionless | Crop coefficient |
| irrigation_l | L | Confirmed applied volume |
| flow_lpm | L/min | Measured flow |
| pressure_kpa | kPa | Measured network pressure |
| forecast_* | canonical units | Forecast weather features available at t |
| physics_theta_24h | m3/m3 | Layer 1 forward forecast at t+24h |
| target_theta_24h | m3/m3 | Actual observed root-zone moisture at t+24h |
| residual_24h | m3/m3 | target_theta_24h - physics_theta_24h |

# 7. Feature Engineering

# Soil lags
sm_lag_1h, sm_lag_3h, sm_lag_6h, sm_lag_12h, sm_lag_24h

# Trends
sm_delta_6h  = theta_rz(t) - theta_rz(t-6h)
sm_delta_24h = theta_rz(t) - theta_rz(t-24h)

# Rolling features
sm_mean_3h, sm_mean_6h, sm_mean_24h
sm_std_24h

# Rainfall accumulation
rain_sum_6h, rain_sum_24h, rain_sum_72h

# Irrigation history
irrigation_sum_6h, irrigation_sum_24h, irrigation_sum_72h
time_since_irrigation_h

# Forecast aggregates
forecast_rain_0_6h
forecast_rain_6_12h
forecast_rain_12_24h
forecast_temp_mean_24h
forecast_temp_max_24h
forecast_rh_mean_24h
forecast_wind_mean_24h
forecast_radiation_24h
forecast_et0_24h

Feature hygiene: Do not create a rolling window that accidentally crosses a train/test boundary using future observations. Generate features on the full chronological stream only from data <= t, then split the resulting timestamped rows chronologically.

# 8. Physics-Forward Baseline (Model 0.5)

This baseline is mandatory. It answers whether XGBoost adds predictive value beyond simply running Layer 1 forward with the same forecast weather.

At time t:
1. Read current measured root-zone state.
2. Load forecast weather available at t for t..t+24h.
3. Run Layer 1 hourly water balance forward.
4. Produce theta_phys_24h.
5. Compare theta_phys_24h against actual theta_24h.

Physics error = theta_obs_24h - theta_phys_24h

# 9. Stage-1 Synthetic Data Strategy

Synthetic data is for pipeline development and methodology validation only. It must never be presented as evidence of real-world field accuracy.

| Synthetic component | How to generate | Purpose |
| --- | --- | --- |
| Base trajectory | Run Layer 1 water balance using weather/crop/soil scenarios | Create physically plausible state trajectories |
| Forecast uncertainty | Perturb forecast weather around plausible error distributions | Stress-test forecast dependency |
| Structured ETc bias | Apply a feature-dependent bias, e.g. crop-stage or high-demand multiplier | Creates learnable residual structure |
| Soil/infiltration bias | Make effective infiltration/storage response differ by soil class | Represents model misspecification |
| Sensor drift | Add a slow bias correlated with time since calibration | Tests sensor-quality correction |
| Random measurement noise | Add bounded stochastic noise after structured effects | Simulates sensor noise without making residual purely random |

# Conceptual synthetic actual state
theta_actual = theta_physics
                  + structured_bias(crop, stage, soil, demand, calibration_age)
                  + bounded_noise

residual = theta_actual - theta_physics

# Required validation claim:
# "Model recovers injected structured residual pattern"
# NOT: "Model is accurate on a real farm."

Synthetic-data rule: The structured bias must depend on variables that are actually present in the XGBoost feature set. Otherwise the residual is intentionally unpredictable and XGBoost should correctly learn little or no correction.

# 10. Training and Validation Protocol

| Stage | Data | Purpose |
| --- | --- | --- |
| Train | Earliest chronological block | Fit XGBoost residual model |
| Validation | Next chronological block | Tune hyperparameters / early stopping |
| Test | Final unseen chronological block | One-time final performance evaluation |
| Future field test | New real field/season block | Deployment/generalization test |

Chronological example (adjust after dataset inspection):
train = first 70% of timestamps
validation = next 15%
test = final 15%

Never random-shuffle the time series for the final evaluation.

# 11. Model Benchmark Ladder

| Model | Role | What it tells us |
| --- | --- | --- |
| Persistence | Minimum baseline | Whether any model beats "tomorrow ≈ today" |
| Layer 1 forward forecast | Physics baseline | Whether ML adds value beyond agronomic physics + forecast weather |
| Ridge regression | Linear baseline | Whether the correction is mostly linear |
| Random Forest | Tree baseline | Robust nonlinear benchmark |
| XGBoost | Primary Layer 2 model | Main residual-learning candidate |
| LightGBM | Boosting challenger | Tests whether another boosting implementation improves results |
| LSTM/GRU | Sequence challenger | Only justify if sufficient clean sequential data exists |

# 12. XGBoost V1 Configuration

Start conservatively. Tune after the pipeline is correct; do not optimize hyperparameters before the data and leakage checks are working.

| Parameter | V1 recommendation | Why |
| --- | --- | --- |
| objective | reg:squarederror | Standard regression objective for continuous residual target |
| tree_method | hist | Efficient histogram-based tree construction |
| n_estimators / rounds | Use validation-based early stopping | Avoid unnecessary boosting |
| learning_rate | Start ~0.03–0.08 | Conservative learning |
| max_depth | Start 3–6 | Control complexity on limited data |
| min_child_weight | Tune | Controls small/over-specific leaves |
| subsample | Start ~0.7–0.9 | Regularization |
| colsample_bytree | Start ~0.7–0.9 | Feature subsampling |
| reg_alpha | Tune | L1 regularization |
| reg_lambda | Tune | L2 regularization |
| early_stopping_rounds | Use validation set | Stop before overfit |

Objective: XGBoost documentation lists reg:squarederror as the standard squared-loss regression objective. It also supports MAE, pseudo-Huber and quantile objectives; these are alternatives to evaluate only if the residual distribution/outlier behavior justifies them.

# 13. Evaluation Metrics

| Metric | Use |
| --- | --- |
| MAE | Primary, easy-to-explain average absolute moisture error |
| RMSE | Penalizes large forecast mistakes |
| R² | Explains variance; secondary only |
| NSE | Hydrologic forecast skill; useful comparison with irrigation/soil-water literature |
| Bias / ME | Detects systematic over/under prediction |
| NRMSE | Normalized comparison across zones/scenarios |
| Threshold error | False/missed crossings of RAW/critical-moisture threshold |
| Post-rain error | Performance during rainfall events |
| High-ET error | Performance during high atmospheric demand |
| Dry-condition error | Performance close to irrigation-trigger region |

Model selection: Do not select the final model solely by R². For the irrigation application, threshold-crossing errors, dry-condition performance, rainfall-event behavior, and missed-irrigation risk matter operationally.

# 14. Fusion and Decision Logic

# Current-state path
Layer 1 current root-zone balance
    -> decide whether irrigation is required now

# Predictive path
Layer 1 forward forecast + XGBoost residual
    -> estimate whether/when the state is likely to cross the irrigation threshold

# V1 safety rule
XGBoost does not directly open the valve.
The deterministic Layer 1/control layer remains the actuator authority.

Example: if Layer 1 says current moisture is safe but the corrected 24-hour forecast falls below the critical threshold, the system should generate a predictive irrigation warning/schedule candidate. The final water volume remains a Layer 1/controller calculation. A pre-emptive automatic action should be enabled only after validation.

# 15. Prediction Quality, Fallbacks and Safety

| Condition | Required response |
| --- | --- |
| Forecast missing/stale | Flag degraded prediction; do not use stale forecast as current truth |
| Sensor missing/invalid | Quality flag; use validated fallback state; do not fabricate a precise value |
| Model residual out-of-distribution | Flag low confidence / fallback to physics-only path |
| Extreme forecast outside training range | Flag extrapolation; prefer conservative deterministic control |
| XGBoost prediction outside physical range | Clip to valid moisture bounds for presentation/control and log the event |
| Large disagreement between physics and ML | Do not blindly average; log disagreement and rely on safety authority |
| Communication loss | Local buffering + deterministic local control path; sync when connection returns |

# 16. Data Logging Contract

| Field | Purpose |
| --- | --- |
| timestamp, zone_id | Traceability |
| raw sensor values | Auditability |
| validated/filtered values | Model input audit |
| forecast issue time + forecast values | Prove what forecast was actually known at prediction time |
| Layer 1 theta_phys_24h | Physics baseline |
| actual theta_24h | Ground-truth observation |
| residual_24h | Training target |
| XGB residual prediction | ML output |
| final corrected prediction | Final forecast |
| model_version | Reproducibility |
| feature_version | Reproducibility |
| prediction_error | Continuous monitoring |
| irrigation command + actual flow/volume | Control verification |

# 17. Implementation Pseudocode

def predict_zone(zone_id, now):
    state = load_current_validated_state(zone_id, now)
    forecast = forecast_provider.get_hourly(zone_id, now, horizon_hours=24)

    if forecast_is_stale_or_incomplete(forecast):
        return degraded_physics_only_path(state)

    physics_forecast = layer1_forward_run(state, forecast, horizon_hours=24)
    features = build_xgb_features(state, forecast, physics_forecast, now)

    residual_hat = xgb_model.predict(features)
    theta_hat_24 = physics_forecast.theta_24 + residual_hat
    theta_hat_24 = clip_to_valid_moisture_range(theta_hat_24)

    decision = layer1_decision_engine(
        current_state=state,
        predicted_theta_24=theta_hat_24
    )

    log_prediction(state, forecast, physics_forecast, residual_hat, theta_hat_24, decision)
    return decision

# 18. Training Pseudocode

# 1. Build timestamped canonical dataset
df = assemble_hourly_zone_data()

# 2. Create features using information available at each timestamp
X = build_features(df)

# 3. Run Layer 1 forward for each timestamp using the forecast that
#    would have been available at that timestamp (historical forecast archive
#    is required for a strict real-world backtest; do not use hindsight forecasts)
theta_phys = build_physics_forward_forecasts(df, historical_forecasts=True)

# 4. Join observed future target
y = df.theta_rz.shift(-24)
residual = y - theta_phys

# 5. Drop rows with incomplete history/targets
X, residual = align_and_filter(X, residual)

# 6. Chronological split
X_train, X_val, X_test, y_train, y_val, y_test = chronological_split(X, residual)

# 7. Fit XGBoost with validation-based early stopping
model = train_xgboost(X_train, y_train, X_val, y_val)

# 8. Evaluate on untouched test period
metrics = evaluate(model, X_test, y_test)

Critical backtesting rule: For a true historical simulation, the physics baseline and forecast features must be generated from the forecast information that would have been available at the prediction timestamp. Substituting later/realized weather or a reprocessed forecast can introduce hindsight leakage.

# 19. Build Order — Do This in This Sequence

1. Implement/verify the Layer 1 forward simulation as a callable function.

2. Integrate a weather-forecast provider through a provider interface; validate timestamps, units and missing fields.

3. Build the canonical hourly database schema for one zone.

4. Implement feature engineering and unit tests for every lag/rolling/forecast feature.

5. Build the physics-forward +24 h baseline first.

6. Build the structured synthetic trajectory generator with known feature-correlated biases.

7. Verify XGBoost can recover those injected residual patterns; report this only as methodology validation.

8. Add public/historical data and, where possible, archived forecasts for realistic backtesting.

9. Train persistence/Ridge/RF/XGBoost/LightGBM benchmarks on the same chronological splits.

10. Evaluate MAE/RMSE/R2/NSE/bias and irrigation-threshold errors.

11. Only then consider LSTM/GRU as a challenger.

12. After hardware deployment, replace/augment synthetic training with genuine field residuals and rerun the full evaluation.

# 20. What We Can Honestly Claim

| Stage | Defensible claim |
| --- | --- |
| Prototype methodology | We have a physics-forward baseline and an XGBoost residual-learning pipeline. |
| Synthetic validation | We can show the model recovers deliberately injected, feature-correlated residual patterns. |
| Public-data validation | We can report performance on a documented public dataset after careful spatial/temporal alignment and leakage controls. |
| Field validation | Only after real sensor/irrigation data has been collected should we claim field-specific predictive performance. |

# 21. Technical References / Implementation References

These references support implementation principles; they do not replace field validation.

XGBoost documentation — Parameters and regression objectives: https://xgboost.readthedocs.io/en/stable/parameter.html

XGBoost documentation — Python API: https://xgboost.readthedocs.io/en/stable/python/python_api.html

Open-Meteo — Forecast API: https://open-meteo.com/en/docs

FAO-56 / crop evapotranspiration guidance: https://www.fao.org/4/X0490E/x0490e00.htm

Status: Layer 2 concept is locked for V1. The next engineering task is not model tuning; it is implementing the forecast dependency, the Layer 1 forward-run interface, the canonical dataset, and the structured synthetic residual generator.